# Ubuntu 22.04 VSD RTL + OpenLane Reinstallation Guide

Purpose: Recreate the working VSD RTL/OpenLane environment established in this session, with commands and expected/observed outputs. Commands are BLUE and outputs are GREEN.

Important: No installation guide can honestly guarantee zero future errors because repositories, package versions, mirrors and Docker images can change. This guide uses the exact versions and fixes observed in this session and adds verification after each major stage. Do not proceed to the next stage until its verification succeeds.

# 1. Baseline Ubuntu packages

$ sudo apt update

Reading package lists... Done

$ sudo apt install -y build-essential git curl wget ca-certificates make gcc g++ cmake autoconf automake libtool pkg-config tcl tcl-dev tk tk-dev tcllib

Installation should complete without errors.

NOTE: Installing tcllib is important: OpenLane v1.0.2 requires Tcl's json package.

# 2. Verify Tcl JSON package

$ tclsh

%

$ package require json

1.3.4

$ exit

vboxuser@Hitesh:~$

# 3. Docker installation and verification

$ sudo apt update

$ sudo apt install -y docker-ce docker-ce-cli containerd.io

docker-ce, docker-ce-cli and containerd.io are installed/up to date.

$ sudo systemctl status docker

Active: active (running)

$ sudo docker run hello-world

Hello from Docker!

This message shows that your installation appears to be working correctly.

$ docker run hello-world

Hello from Docker!

NOTE: The second command verifies that the normal user can access Docker without sudo.

# 4. Clone OpenLane

$ cd ~

$ git clone https://github.com/The-OpenROAD-Project/OpenLane.git

Cloning into 'OpenLane'...

Receiving objects: 100% ...

$ cd ~/OpenLane

$ pwd

/home/vboxuser/OpenLane

$ ls

AUTHORS.md  designs  flow.tcl  LICENSE  requirements.txt  configuration  docker  scripts  tests ...

# 5. Build OpenLane

$ make openlane

The OpenLane container image is pulled and the Python virtual environment is created.

$ docker images | grep openlane

ghcr.io/the-openroad-project/openlane:ff5509f65b17bfa4068d5336495ab1718987ff69-amd64 ...

$ ~/OpenLane/venv/bin/python3 --version

Python 3.10.12

$ ~/OpenLane/venv/bin/ciel --version

Ciel v2.6.1 ©2022-2025 Efabless Corporation and Contributors

$ ~/OpenLane/venv/bin/python3 -m pip check

No broken requirements found.

# 6. Enable Sky130A PDK

$ cd ~/OpenLane

$ make pdk

./venv/bin/ciel enable --pdk sky130A

Version 0fe599b2afb6708d281543108caf8310912f54af enabled for the sky130 PDK.

$ ls -la ~/.ciel

sky130A -> ciel/sky130/versions/0fe599b2afb6708d281543108caf8310912f54af/sky130A

$ ls -la ~/.ciel/ciel/sky130/versions/0fe599b2afb6708d281543108caf8312f54af/sky130A

.config  libs.ref  libs.tech  SOURCES

# 7. Verify OpenLane container

$ docker run --rm ghcr.io/the-openroad-project/openlane:ff5509f65b17bfa4068d5336495ab1718987ff69-amd64 ls /

... openlane ... tool_metadata.yml ...

$ docker run --rm ghcr.io/the-openroad-project/openlane:ff5509f65b17bfa4068d5336495ab1718987ff69-amd64 cat /tool_metadata.yml

Metadata lists cvc_rv, magic, netgen, padring, vlogtoverilog, verilator, yosys, klayout, openroad_app, git and open_pdks.

$ docker run --rm ghcr.io/the-openroad-project/openlane:ff5509f65b17bfa4068d5336495ab1718987ff69-amd64 /openlane/flow.tcl -version

OpenLane v1.0.2 (ff5509f65b17bfa4068d5336495ab1718987ff69)

NOTE: Do NOT use /openlane/bin/openlane; that path does not exist in this image.

# 8. Start the OpenLane container

$ cd ~/OpenLane

$ make mount

OpenLane Container (ff5509f):/openlane$

NOTE: The working flow method is to run OpenLane inside this container. Direct host ./flow.tcl -version may report 'Container manifest not found'.

# 9. Run the SPM test design

$ ./flow.tcl -design spm

[INFO]: Using configuration in 'designs/spm/config.json'...

[INFO]: Process Design Kit: sky130A

[INFO]: 0 errors found by linter

[INFO]: No DRC violations after detailed routing.

[INFO]: No XOR differences between KLayout and Magic gds.

[INFO]: No DRC violations after GDS streaming out.

[INFO]: There are no hold violations in the design at the typical corner.

[INFO]: There are no setup violations in the design at the typical corner.

[SUCCESS]: Flow complete.

NOTE: The successful session also produced warnings: linter warning; small core/power-grid scaling; STA blackboxes for tap/decap/fill cells; VSRC_LOC_FILES not defined; max fanout violations. These warnings did not stop the flow. Inspect reports before treating the result as signoff-clean.

$ exit

vboxuser@Hitesh:~/OpenLane$

# 10. VSD RTL tools on the Ubuntu host

## Yosys

$ yosys -V

Yosys 0.xx (record the actual installed version)

## Icarus Verilog

$ sudo apt update

$ sudo apt install -y iverilog

$ iverilog -V

Icarus Verilog version ...

## GTKWave 3.3.86

$ cd ~/Downloads

$ wget https://archive.ubuntu.com/ubuntu/pool/universe/g/gtkwave/gtkwave_3.3.86-1_amd64.deb

$ ls -lh gtkwave_3.3.86-1_amd64.deb

... 2.3M ... gtkwave_3.3.86-1_amd64.deb

$ sudo apt install ./gtkwave_3.3.86-1_amd64.deb

Setting up gtkwave (3.3.86-1) ...

$ gtkwave --version

GTKWave Analyzer v3.3.86

NOTE: If apt prints the 'Download is performed unsandboxed as root' N: notice for a local .deb, it is not an installation error.

## OpenSTA

$ sta -version

Record the installed version.

## ngspice

$ ngspice -v

Record the installed version.

## Magic

$ magic --version

Record the installed version.

# 11. GTKWave rendering problem in VirtualBox

NOTE: In this session GTKWave launched but the GUI was incorrectly rendered. Reinstalling 3.3.86 did not fix it, indicating the issue may be VirtualBox/graphics rather than the GTKWave package.

$ LIBGL_ALWAYS_SOFTWARE=1 gtkwave

NOTE: If this fixes the GUI, investigate VirtualBox Display/3D acceleration. Do not repeatedly reinstall GTKWave without testing rendering.

$ gtkwave ~/VLSI/sky130RTLDesignAndSynthesisWorkshop/verilog_files/tb_good_mux.vcd

GTKWave Analyzer v3.3.86

[0] start time.

[300000] end time.

NOTE: A 'Failed to load module canberra-gtk-module' message is a warning and does not by itself indicate GTKWave failure.

# 12. GVim

$ sudo apt update

$ sudo apt install -y vim-gtk3

$ gvim --version

VIM - Vi IMproved 8.2 ...

Huge version with GTK3 GUI

$ gvim -p file1.v file2.v file3.v

NOTE: Quit without saving: Esc, :q!, Enter. Quit all without saving: :qa!.

# 13. Notepad++

$ sudo snap install notepad-plus-plus

notepad-plus-plus installed

$ notepad-plus-plus

NOTE: Notepad++ is a Windows application packaged for Linux through a compatibility layer.

$ notepad-plus-plus file1.v file2.v file3.v

# 14. LIB_FOLDER

$ export LIB_FOLDER="$(pwd)"

$ echo "$LIB_FOLDER"

/home/vboxuser/<current-directory>

$ cd "$LIB_FOLDER"

$ pwd

/home/vboxuser/<current-directory>

NOTE: If LIB_FOLDER is placed in .bashrc, prefer a fixed absolute path rather than $(pwd) if you need deterministic behavior.

# 15. Bash history and terminal dumps

$ history

$ less ~/.bash_history

$ history -w

$ cp ~/.bash_history ~/bash_history_$(date +"%Y-%m-%d_%H-%M-%S").txt

NOTE: clear or Ctrl+L only clears the visible screen; it does not erase Bash history.

$ mkdir -p ~/terminal_dumps

$ script ~/terminal_dumps/terminal_dump_$(date +"%Y-%m-%d_%H-%M-%S").txt

Script started, output log file is '/home/vboxuser/terminal_dumps/terminal_dump_YYYY-MM-DD_HH-MM-SS.txt'.

NOTE: script captures terminal control sequences. If you need a clean text copy, keep the raw file and clean a separate copy.

# 16. Bulk cleaning terminal dumps

NOTE: Create the script with Notepad++:

$ notepad-plus-plus ~/terminal_dumps/clean_terminal_dumps.sh

Paste/save the following:

#!/bin/bash
DUMP_DIR="$HOME/terminal_dumps"
for file in "$DUMP_DIR"/*.txt; do
    [[ "$file" == *_clean.txt ]] && continue
    [[ -e "$file" ]] || continue
    output="${file%.txt}_clean.txt"
    echo "Cleaning: $(basename "$file")"
    echo "      -> $(basename "$output")"
    perl -pe '
        s/\e\[[0-9;?]*[ -\/]*[@-~]//g;
        s/\e\][^\a]*(?:\a|\e\\)//g;
        s/\r//g;
        s/[\x00-\x08\x0B\x0C\x0E-\x1F\x7F]//g;
    ' "$file" > "$output"
done
echo "Cleaning completed."

$ chmod +x ~/terminal_dumps/clean_terminal_dumps.sh

$ ~/terminal_dumps/clean_terminal_dumps.sh

Cleaning completed.

# 17. ZIP backup

$ sudo apt update

$ sudo apt install -y zip

$ cd ~/VLSI

$ zip -r ~/sky130RTLDesignAndSynthesisWorkshop.zip sky130RTLDesignAndSynthesisWorkshop

adding: sky130RTLDesignAndSynthesisWorkshop/ ...

$ ls -lh ~/sky130RTLDesignAndSynthesisWorkshop.zip

$ unzip -l ~/sky130RTLDesignAndSynthesisWorkshop.zip

# 18. Common errors already encountered and their fixes

Bash: package: command not found
package require json must be run inside tclsh. Install tcllib, run tclsh, then package require json.

OpenLane: Container manifest not found
Run make mount and execute ./flow.tcl -design <design> inside the OpenLane container.

No /openlane/bin/openlane
Use /openlane/flow.tcl; the container image does not contain /openlane/bin/openlane.

make list / make help: No rule
Those targets are not present in this Makefile. Inspect targets with grep -E '^[a-zA-Z_-]+:' Makefile.

GTKWave GUI corrupted
Test LIBGL_ALWAYS_SOFTWARE=1 gtkwave; investigate VirtualBox graphics/3D acceleration if this fixes it.

canberra-gtk-module warning
Optional GTK module warning; it does not mean GTKWave failed.

GitHub password authentication failure
Use SSH or a GitHub Personal Access Token; do not use the normal GitHub account password.

script dump contains special characters
script records raw terminal control sequences; preserve raw log and create a cleaned copy with the supplied script.

# 19. Final verification checklist

docker run hello-world works without sudo.

tclsh → package require json returns 1.3.4.

~/OpenLane/venv/bin/python3 -m pip check returns No broken requirements found.

~/OpenLane/venv/bin/ciel --version reports Ciel v2.6.1 in the recorded environment.

make pdk reports Sky130A enabled.

make mount enters the OpenLane container.

Inside the container, ./flow.tcl -design spm ends with [SUCCESS]: Flow complete.

yosys -V works.

iverilog -V works.

gtkwave --version reports 3.3.86 for the target installation.

sta -version works.

ngspice -v works.

magic --version works.

gvim --version works.

notepad-plus-plus launches if installed.

# 20. Version snapshot from the successful session

OpenLane: 1.0.2

OpenLane container tag: ff5509f65b17bfa4068d5336495ab1718987ff69-amd64

Ciel: 2.6.1

Python in OpenLane venv: 3.10.12

Tcl JSON package: 1.3.4

GTKWave target installation: 3.3.86

Sky130A Ciel version: 0fe599b2afb6708d281543108caf8310912f54af